Formula List for Fiber Optics

- Numerical Aperture (N.A.)

N.A.=sinθ0=μ12-μ22

Where;

- θ0    = Acceptance angle

- μ1    = Refractive index of core

- μ2    = Refractive index of cladding

- Fractional Refractive Index (∆)

∆=μ1-μ2μ1

Where;

- μ1    = Refractive index of core

- μ2    = Refractive index of cladding

- Numerical Aperture in terms of  ∆

N.A.=μ12∆

Where;

- μ1    = Refractive index of core

- ∆    = Fractional refractive index of the fibre

- Critical angle at core-cladding interface (φc)

φc=sin-1μ2μ1

Where;

- μ1    = Refractive index of core

- μ2    = Refractive index of cladding

- Normalized Frequency (V)

V=2πaגμ12-μ22

Where;

- μ1    = Refractive index of core

- μ2    = Refractive index of cladding

- ג   = Wavelength travelling through the fiber

- a   = Radius of the core

- Number of modes (Nm)

Nm=V22

Where;

- V    = Normalized Frequency